B.Sc. (H) Physics IV Semester

UPC- 32221403

Paper Name – Analog Systems and Applications

Topic – RC Coupled Amplifier and its Frequency Response (4 lectures)

- Understanding the working of Single Stage Common Emitter Amplifier:

Click on:  https://youtu.be/1SX75vWfQNw

- Frequency response of Single Stage Common Emitter Amplifier:

Click on: https://youtu.be/hFlyThTeff0

- Working of Two Stage RC Coupled Amplifier:

Click on: https://youtu.be/WO0cTfg5B_Q

The frequency response of two stage amplifier is as follows. The roll off (or cut off) frequencies are marked at the 3dB gain (i.e. 70% of maximum gain).

Important characteristics of an RC coupled amplifier:

- Used as an audio frequency (AF) amplifier

- It has a wide bandwidth

- Flat output with very less distortion

- Low gain at lower frequencies is due to coupling capacitors.

- Low gain at higher frequencies is due to wiring capacitances and short circuited shunt capacitances.

Advantages:

1. Inexpensive coupling method using only capacitor.

2. Large bandwidth.

3. Low distortion, flatter region.

4. Q point and DC bias conditions do not change.

Disadvantages:

1. As the gain reduces in LF/HF range, it can't be uses to amplify low/high frequencies.

2. Overall less gain (because of wide BW)

3. Low impedance matching

4. Becomes noisy with age.

Applications: RC coupled amplifiers are used in

1. Tape Recorders

2. TV, VCR, CD player

3. Stereo Amplifier

4. AF Amplifiers

-Dr Shivani Agarwal